Implementation & Evaluation Plan

AI-Based Building-Cluster Flood Vulnerability Mapping

Using Satellite Imagery and Geospatial Data (Kochi / Periyar Basin, Kerala)

Version 2 — revised to address building-level resolution limits, single-source validation risk, and scoring scope

Aligned against base paper: Mohamadiazar et al. (2024), Journal of Hydrology

# 1. Scope Statement — Read This First

This project is specifically about flood risk, not general urban-planning risk. We do not evaluate traffic, amenities, seismic risk, or general "quality of planning." Every dataset, every metric, and every score in this document measures one thing: how vulnerable a location is to flooding, based on elevation, water proximity, land cover, and historical flood evidence. We use the precise term

"Flood Vulnerability Score" throughout this document — never "flood prediction" and never "risk" used vaguely — because we are estimating relative vulnerability from known contributing factors, not forecasting the probability of a future event.

# 2. What Changed From Version 1, and Why

| Version 1 (earlier plan) | Version 2 (this document) | Why it changed |
| --- | --- | --- |
| Scored only newly detected construction | Scores every building/land unit in the study area; new construction becomes an overlay tag | Restricting to new construction discarded most of the segmentation output and made the citizen "check before building" feature not actually work on empty land |
| Building-level risk claims | Grid-cell / building-cluster level claims | Sentinel-2 is 10 m resolution — a single house is often smaller than one pixel. Claiming individual building precision overclaims what the data can support |
| DFO 2018 as the sole ground truth | DFO 2018 + supplementary Sentinel-1-derived flood extent + local records, used together | DFO flood products are relatively coarse (regional-scale, documented resolution limitations); a single coarse source cannot be treated as precise ground truth on its own |
| Two land-cover classes (building, vegetation) | Five land-cover classes (building, vegetation, water, impervious surface, bare land) | More classes give richer, more defensible risk factors (e.g. imperviousness as a runoff proxy) instead of relying on only two |
| "Risk score" used loosely | "Flood Vulnerability Score", explicitly not a probability | Avoids implying a calibrated probability of flooding without the historical event data that would justify one |
| Precision/Recall/F1/AUC only | Adds IoU and Dice coefficient for segmentation; adds ROC-AUC/PR-AUC framing for risk validation | Pixel accuracy and F1 alone are not sufficient for segmentation quality; PR-AUC matters because flooded/high-risk areas are a minority class |

# 3. Research Question

"Can satellite-based land-cover segmentation, multi-year change detection, and geospatial hazard factors be combined to estimate the relative flood vulnerability of buildings and building clusters, and to identify newly developed areas that fall within potentially vulnerable zones?"

This framing — vulnerability estimation and growth analysis, not flood prediction — is what we present to the guide and panel.

# 4. Implementation Plan

## 4.1 Stage 1 — Data Collection & Alignment

| Layer | Source | Resolution | Role |
| --- | --- | --- | --- |
| Optical satellite imagery (2 years) | Sentinel-2 Level-2A, AWS Earth Search STAC | 10 m, bands B02/B03/B04/B08 | Land-cover segmentation; change detection |
| Radar satellite imagery (supplementary) | Sentinel-1 SAR, Copernicus | 10 m | Cloud-penetrating cross-check for Kerala's monsoon cloud cover; supplementary flood-extent signal for validation |
| Elevation | Copernicus Global DEM (GLO-30) | ~30 m | Low-lying zone identification (relative elevation, not exact per-building height) |
| Water bodies / rivers | OpenStreetMap hydrography | Vector | Distance-to-water computation |
| Building footprints (labels) | OSM + Microsoft Global ML Building Footprints + Google Open Buildings v3 | Vector polygons | Training/testing labels for segmentation; building-cluster definition |

All layers are resampled onto one shared grid and coordinate system before further processing. Sentinel-1 is included specifically because Kerala's monsoon-heavy climate frequently produces cloud cover that blocks optical Sentinel-2 imagery — Sentinel-1's radar sees through cloud and works day or night.

## 4.2 Stage 2 — Multi-Class Land-Cover Segmentation

- Model: DeepLabV3+ (or U-Net), pretrained on ImageNet, fine-tuned on local labels

- Classes (5, not 2): Building, Vegetation, Water, Impervious surface / road, Bare land

- Unit of analysis: building clusters / fixed grid cells aligned to imagery resolution — not individual buildings, to avoid overclaiming precision below the sensor's real resolution

- Train/test split: 70% / 30%, with data augmentation (flip, rotation) — same convention as the base paper

## 4.3 Stage 3 — Multi-Year Change Detection

The trained model is run on the earlier-year and later-year image sets, and outputs are compared per grid cell / cluster to flag newly developed or expanded built-up clusters. This module answers "where has growth happened" and feeds a growth-trend tag into the scoring stage — it is not the gatekeeper for whether a location gets scored at all.

## 4.4 Stage 4 — Geospatial Feature Extraction

For every grid cell / building cluster in the study area (not only new construction), we compute:

- Elevation and relative elevation (Copernicus DEM)

- Distance to nearest water body (OpenStreetMap)

- Local slope (derived from DEM)

- Imperviousness proxy (share of impervious/road class in and around the cell — a runoff indicator)

- Vegetation share (a partial drainage/absorption indicator)

- Historical flood exposure (whether this cell fell inside a documented past flood extent)

## 4.5 Stage 5 — Weighted Flood Vulnerability Scoring

Flood Vulnerability Score =
    w1 x elevation_risk
  + w2 x water_proximity_risk
  + w3 x historical_flood_exposure
  + w4 x impervious_surface_share
  + w5 x slope_risk
  + w6 x (1 - vegetation_share)
 
Normalized to a 0-100 scale:
  0-20   Very Low
  21-40  Low
  41-60  Moderate
  61-80  High
  81-100 Very High

Initial weights (w1...w6) are assigned manually based on established flood-hydrology literature (elevation and water proximity weighted highest, consistent with the base paper's own finding that these factors matter most). This is disclosed explicitly as a heuristic starting point. As a stretch goal, weights can later be learned from data (e.g. logistic regression against the historical flood-extent labels) instead of set manually — noted as a future extension, not a current claim.

Every scored cell/cluster also carries a factor-level breakdown ("why is this score high") for explainability, and a separate "recently developed" tag from Stage 3 where relevant — the growth-trend overlay.

## 4.6 Stage 6 — Validation

Vulnerability scores are compared against real historical flood evidence for the 2018 Kerala floods, combining more than one independent source rather than relying on a single dataset (see Section 6).

## 4.7 Stage 7 — Dashboard

A planner/authority view (zone-level vulnerability map with growth-trend overlay) and a citizen-facing lookup (plain-language, explainable summary for any searched location, including empty/undeveloped land) are both served from the same underlying score.

# 5. Implementation Block Diagram

# 6. Validation — Ground Truth, Handled Honestly

## 6.1 Why DFO Alone Is Not Enough

DFO's flood-extent products are documented as relatively coarse (regional-scale, on the order of ~250 m in places), and DFO itself notes that not all flooding is reliably mappable from satellite data. Treating a DFO polygon as proof that one specific building or cluster flooded would be an overclaim. We therefore use DFO as one of several independent evidence sources, not as the sole ground truth.

## 6.2 Our Multi-Source Validation Approach

| Source | What it adds | Resolution / caveat |
| --- | --- | --- |
| Dartmouth Flood Observatory (DFO), 2018 Kerala event | Independent, publicly documented regional flood-extent polygon | Coarser resolution; regional-scale, not per-building |
| Sentinel-1 SAR-derived flood extent (supplementary, self-derived if feasible) | A finer-resolution, radar-based cross-check for the same 2018 event | Depends on scene availability for the exact flood dates |
| Local records / news coverage of the 2018 floods | Qualitative cross-check for specific known-affected localities | Not systematic or exhaustive; used as a sanity check only |

## 6.3 How the Comparison Works (Grid-Cell / Cluster Level)

- Overlay our Flood Vulnerability Score map on the same grid as the combined flood-evidence layer

- For each cell/cluster, treat "did this location fall inside documented flood extent" as a binary label

- Compare against our vulnerability score using both a threshold-based Risk Validation Rate and a threshold-free ROC-AUC / PR-AUC curve

Risk Validation Rate = (High/Very-High cells overlapping documented flood extent
                         / Total High/Very-High cells flagged) x 100%
 
PR-AUC used alongside ROC-AUC because flooded cells are a minority class
(class imbalance) - PR-AUC is more informative than ROC-AUC alone in this case

This is the direct structural equivalent of the base paper's Ground Truth Index (84.05%), but built from multiple evidence sources and reported at the resolution our data actually supports, not at an implied building-level precision.

# 7. Evaluation Parameters

## 7.1 Segmentation Quality

| Metric | Formula / meaning | Why included |
| --- | --- | --- |
| Pixel Accuracy | (TP+TN)/(TP+TN+FP+FN) | Overall correctness, comparable to base paper |
| Precision | TP/(TP+FP) | Comparable to base paper |
| Recall | TP/(TP+FN) | Comparable to base paper |
| F1 Score | 2 x P x R / (P+R) | Comparable to base paper |
| IoU (Intersection over Union) | Overlap / union of predicted vs. true region | Standard, stricter segmentation metric — added per methodological review |
| Dice coefficient | 2 x ¦A intersect B¦ / (¦A¦+¦B¦) | Standard for imbalanced segmentation classes (e.g. small building footprints) — added per methodological review |

## 7.2 Change Detection

Evaluated by manual sample verification against Google Earth historical imagery (no formal automated dataset exists for this step, disclosed openly), reporting Precision/Recall/F1/IoU on the manually verified sample of flagged clusters.

## 7.3 Flood Vulnerability Scoring

| Metric | Purpose |
| --- | --- |
| Risk Validation Rate | Direct equivalent of base paper's Ground Truth Index, using our combined evidence layer |
| ROC-AUC | Threshold-independent separability between vulnerable and non-vulnerable cells |
| PR-AUC | More informative than ROC-AUC given flooded/high-risk cells are a minority class |
| Confusion matrix | Full breakdown of the binary high-risk vs. not comparison at a chosen threshold |
| Ablation study (per weight factor) | Tests which of the six weighted factors actually drive validation performance |

# 8. Ablation Study

| Case | Factors included | Purpose |
| --- | --- | --- |
| 1 | Elevation only | Isolate elevation's contribution |
| 2 | Water proximity only | Isolate water proximity's contribution |
| 3 | Elevation + water proximity | Core two-factor baseline (matches Version 1 logic) |
| 4 | + Imperviousness | Test runoff-proxy contribution |
| 5 | + Vegetation + slope + historical exposure (full model) | Complete weighted model — expected best performance |

Mirrors the base paper's own 8-case input-combination study (their Table 4), testing rather than assuming that more factors improve validation performance.

# 9. Limitations We State Openly

- Resolution limit: Sentinel-2's 10 m pixels cannot reliably delineate individual small buildings — we score at grid-cell/cluster level, not per-building, and say so explicitly in the report

- Cloud cover: optical Sentinel-2 imagery can be affected by Kerala's monsoon cloud cover — mitigated by using Sentinel-1 radar as a supplementary, cloud-independent source

- DEM resolution: ~30 m DEM does not capture exact per-building elevation — we describe it as terrain/relative-elevation context around a cluster, not an exact height

- Flood risk is multi-causal: rainfall intensity, drainage infrastructure, river discharge, tides, and soil permeability all affect real flooding — our score captures the terrain/land-cover factors it has data for, not the full causal picture

- DFO is not building-level ground truth: used as one of several evidence sources, not sole proof, as detailed in Section 6

- Change detection can misfire on clouds, shadows, seasonal vegetation change, or construction materials — mitigated by manual sample verification, not treated as error-free

- The Flood Vulnerability Score is a relative estimate, not a calibrated probability of flooding — this distinction is maintained in all reporting language

# 10. Scoping — MVP and Extensions

The full design above is ambitious for a single semester. We define a clear core deliverable and treat the rest as disclosed extensions, not silent scope-cutting:

Core deliverable (MVP): Kochi-area (Aluva / Periyar basin) multi-class land-cover segmentation (Sentinel-2 only) + multi-year change detection + elevation/water-proximity-based vulnerability scoring + DFO validation + interactive map.

- Extension 1: Add Sentinel-1 radar fusion for cloud-robustness

- Extension 2: Add imperviousness, slope, and vegetation as additional scoring factors

- Extension 3: Add Sentinel-1-derived supplementary flood-extent validation alongside DFO

- Extension 4: Learn scoring weights from data instead of manual assignment

- Extension 5: Public-facing citizen dashboard with full explainability breakdown

# 11. Novelty vs. Base Paper (Mohamadiazar et al., 2024) — Point by Point

- Scope: base paper predicts near-real-time flood extent during/after a live rainfall event (requires live rainfall data as input); we produce a standing flood-vulnerability map independent of any specific storm, usable at any time, including before construction occurs.

- Temporal design: base paper is a single time-snapshot prediction per image/date; we add explicit multi-year change detection layered on top of vulnerability scoring, tracking where risk is actively growing over time.

- Segmentation richness: base paper performs binary water/non-water detection from radar backscatter; we perform 5-class land-cover segmentation (building, vegetation, water, impervious surface, bare land) from optical imagery, giving richer, more defensible risk factors such as imperviousness.

- Explainability: base paper outputs a flooded/non-flooded pixel mask with no per-factor breakdown; our score is a weighted combination of named factors, each shown to the end user ("why is this score high"), addressing the interpretability gap the base paper's own literature review flags as a limitation of many ML flood models.

- Validation approach: base paper validates against a single structured source (311 complaint database + Twitter); we combine multiple independent flood-evidence sources (DFO + supplementary Sentinel-1-derived extent + local records) specifically because our primary source (DFO) is coarser than a structured complaint database, and disclose this reasoning openly.

- Audience: base paper targets near-real-time emergency response and infrastructure operations; we add a citizen-facing "check my area / check before building" tool alongside a planner dashboard, extending decision support beyond emergency responders to the general public.

- Geographic and climate context: base paper is built and tested on Miami-Dade County, Florida (temperate coastal US climate); we fine-tune specifically for Kerala's monsoon-driven, cloud-heavy tropical context, addressing a real representativeness gap in existing literature, and specifically add Sentinel-1 to handle monsoon cloud cover, a constraint the base paper's US context did not need to solve.

- Resolution honesty: base paper works at 10 m pixel resolution matched to its radar water-detection task; we explicitly score at grid-cell/building-cluster resolution rather than claiming individual-building precision that Sentinel-2's resolution cannot actually support — a methodological discipline point, not just a technical difference.

# 12. One-Paragraph Summary (For Quick Reference)

"We build a flood-specific vulnerability mapping system for the Kochi/Periyar basin area, combining multi-class land-cover segmentation, multi-year change detection, and a weighted, explainable scoring framework over elevation, water proximity, imperviousness, slope, vegetation, and historical flood exposure. Scoring is performed at grid-cell/building-cluster resolution to honestly match Sentinel-2's spatial resolution, not at an overclaimed per-building precision. Validation combines the Dartmouth Flood Observatory's 2018 Kerala flood-extent data with supplementary Sentinel-1-derived evidence and local records, avoiding reliance on a single coarse ground truth. The system is explicitly a Flood Vulnerability Score, not a flood-prediction probability, and is designed for two audiences: planners monitoring growth in vulnerable zones, and citizens checking flood vulnerability for any location, including before construction."

- End of document -

# PART II — VERSION 3 (Revised Research Design)

Version 3 keeps everything Version 2 got right — the flood-specific scope, the honest language about resolution, and the multi-source validation — and changes the research core. Version 2 was a sound engineering plan but a weak research contribution, because its central method (a hand-weighted scoring formula) has already been published for Kerala, and published as the weaker of two options. Version 3 replaces that core with a question none of the four reference papers answers, while staying fully buildable in one semester with free, public data.

# 13. Proposed Title

Primary title

“Development-aware flood vulnerability mapping for pre-construction decision support: A flow-coupled, positive-unlabeled machine learning framework for the Periyar basin, Kerala”

Alternate titles (same idea, different emphasis)

- “Prospective flood vulnerability mapping under urban growth: An explainable, flow-coupled machine learning framework for Kochi, Kerala”

- “From where it flooded to where it will flood: Development-aware flood susceptibility mapping using satellite imagery, flow routing and positive-unlabeled learning”

Short name for the project: ProFlood-Kerala (Prospective Flood vulnerability).

# 14. Why Version 3 Was Needed — An Honest Gap Audit

Before writing Version 3 we checked, item by item, whether each of our Version 2 novelty claims was actually new. Most were not. Stating this openly is deliberate: a panel will run the same check, and it is far better that we run it first.

## 14.1 Our Version 2 claims versus what already exists

| Version 2 claim | What the literature already has | Verdict |
| --- | --- | --- |
| Weighted vulnerability score over elevation, water proximity, imperviousness, slope, vegetation | Kendagannaswamy et al. (2025) already do exactly this for Kerala: ten factors, AHP-derived weights, FSI = Σ(Wi × Ci). They also benchmark it and find it worse than LightGBM (AUC 0.763 vs 0.948). | Not novel, and the weaker method |
| Per-factor explainability (“why is this score high”) | Antzoulatos et al. (2022) and Waleed & Sajjad (2025) both publish SHAP-based explainable flood models. This is current standard practice. | Not novel |
| Five-class land-cover segmentation from Sentinel-2 | ESA WorldCover and Google Dynamic World already provide 10 m global land cover for free. Re-training a model to reproduce them is a good exercise, not a finding. | Not novel |
| Hazard combined with exposure to give risk | Antzoulatos et al. compose Risk = f(Hazard, Vulnerability, Exposure); Waleed & Sajjad add population and economic exposure. | Not novel |
| New study area (Kerala) and citizen-facing audience | A new region and a new user interface are scope and engineering. A panel grading novelty will not count them as a research contribution. | Not sufficient |
| Multi-year change detection as a growth overlay | Susceptibility change under land-cover change has been published (e.g. a Kuala Lumpur 2000-vs-2020 comparison). A two-snapshot descriptive comparison is now routine. | Not novel as an overlay |

## 14.2 Methodological upgrades we checked — and found already taken

We also checked the obvious “add a better method” routes so that we do not spend a semester rediscovering published work:

- Spatial cross-validation to remove the accuracy inflation caused by spatial autocorrelation — now an active area in susceptibility mapping. Random splits over raster data leak neighbouring pixels into the test set and inflate AUC by roughly 5–15 %, which is why published values cluster at 0.94–0.99.

- Positive-unlabeled (PU) learning for the “non-flood points are not really negatives” problem — already published for urban flood susceptibility (Land, 2022).

- Conformal uncertainty on flood susceptibility in India — already published (flash-flood mapping in Himachal Pradesh with conformal prediction).

- Counterfactual what-if explanations for flood susceptibility — already touched (São Paulo, LIME plus counterfactual analysis, 2025).

Conclusion: no single method swap is novel on its own. The novelty has to come from a question nobody is asking, with these methods used as the tools that answer it. Section 16 states that question.

# 15. Limitations and Future Scope of the Four Reference Papers — and How V3 Uses Them

This section is the backbone of Version 3. Three of the four papers explicitly name, in their own conclusions, the gap we intend to fill. That is the strongest possible justification we can offer a panel: we are not inventing a problem, we are executing the future work these authors asked for.

## 15.1 Mohamadiazar, Ebrahimian & Hosseiny (2024) — Journal of Hydrology (base paper)

| What the paper states | How Version 3 uses it |
| --- | --- |
| Limitation: uncertainty in matching daily rainfall to the satellite image date. Flooding seen in an image may result from rain that fell the same day, the day before, or earlier. They recommend hourly weather-station monitoring. | We side-step this failure mode entirely by not doing event-time prediction. We build a standing susceptibility map and use rainfall as a long-run statistic (count of days above 10 mm, CHIRPS 2010–2024) rather than a same-day value, so a timing mismatch cannot propagate into the model. |
| Future scope: use Google Earth Engine for preprocessing Sentinel-1 imagery. | Our entire Stage 1 and Stage 2A run in Google Earth Engine, which also removes the manual SNAP workflow the paper had to use. |
| Future scope: keep feeding new Sentinel-1 images and updated DEM/imperviousness layers into the model to keep it a “live tool”. | Our multi-epoch design (2017 / 2018 / 2019 / 2021 / 2024) is exactly this idea — but we turn it from a maintenance suggestion into a measurable test (Section 22, Experiment E1): does a model fitted on 2018 still work in 2021? |
| Not addressed: the model outputs a flooded / not-flooded pixel mask with no per-factor reasoning, and does binary water detection only. | We output a graded susceptibility value with SHAP attribution and a constrained counterfactual telling the user what would change the answer (Stage 6). |
| Not addressed: rainfall is a required live input, so the model cannot say anything about a plot when no storm is happening. | Our map is storm-independent and therefore answerable at any time — which is what a “check before you build” query needs. |

## 15.2 Antzoulatos et al. (2022) — Sustainability (explainable ML framework)

| What the paper states | How Version 3 uses it |
| --- | --- |
| Stated as a crucial issue: the lack of annotated datasets for training flood ML models. They solve it with a rule-based annotation derived from water depth and velocity, which requires an existing calibrated 2D hydraulic model. | No calibrated 2D hydraulic model exists for the Periyar basin, so their solution is not available to us. We therefore attack the label problem directly with positive-unlabeled learning (Stage 4) instead of inventing rules we cannot validate. |
| Limitation: the pixel-based approach makes processing time and memory grow sharply with resolution. | We work at 10 m over a single basin rather than a state, and our sub-watershed blocking scheme also bounds memory per fold. |
| Future work: integrate social-media information into the flood risk algorithm. | Local records and news coverage of the 2018 event are retained from Version 2 as a third corroboration source for the flood inventory (Section 6.2). |
| Stated need: static flood risk maps are tied to predefined return periods that may not match a real event; authorities need a dynamic complement. | Our uplift value Δ and the build-out scenario simulator are exactly that dynamic complement — but driven by land-use change rather than by a live storm, which is the axis their tool does not cover. |
| Finding: flood conditioning factor importance depends on the geomorphology of the area; their ranking is specific to a low-slope coastal setting. | We do not import their weights. We learn factor influence from Kerala data and cross-check SHAP against our own ablation study (Section 23). |

## 15.3 Waleed & Sajjad (2025) — IJDRR (Pakistan, AI + geospatial framework)

| What the paper states | How Version 3 uses it |
| --- | --- |
| Future scope, stated directly: “land-use changes, such as deforestation and urban sprawl, can exacerbate flood risks … Future research could integrate such variables to develop a more holistic framework.” | This is our core contribution. Our UICA and ΔUICA features and the entire development-aware prediction stage are a direct implementation of this stated future direction. |
| Limitation: “relying on historical flood data poses limitations as it may not capture evolving patterns driven by land-use changes and climate change”. | Our multi-epoch inventory plus temporal hold-out measures the decay they warn about, instead of only warning about it (Experiment E1). |
| Limitation: results are sensitive to how the continuous susceptibility value is cut into five classes (they use a quantile scheme). | We report threshold-free ROC-AUC and PR-AUC as the primary metrics, and add a sensitivity analysis across three class-cutting schemes (quantile, natural breaks, fixed cut-offs). |
| Limitation: SHAP and other explainable-AI outputs carry context-dependent biases; feature-importance rankings may be unreliable across heterogeneous areas. | We restrict the study to one hydrologically coherent basin, and we never rely on SHAP alone — every claimed factor contribution must also show up in the ablation study. |
| Limitation: vulnerability properly means exposure plus sensitivity plus adaptive capacity (Adger; IPCC). They could only model exposure. | We keep Version 2’s honest wording: we model the terrain and land-cover side only, and we say so in the report rather than implying a full vulnerability assessment. |
| Ethical note: high-resolution flood data carries a risk of misuse, such as inequitable resource allocation or discriminatory practice. | We add an explicit ethics statement (Section 27) and ship an Area-of-Applicability mask so that low-confidence areas are visibly greyed out rather than presented as fact. |
| Limitation: Google Earth Engine could not host their ML models, forcing a local export. | We adopt the same split by design: Earth Engine for data preparation, local Python for LightGBM / XGBoost training. This is a known-good path, not an unknown. |

## 15.4 Kendagannaswamy et al. (2025) — Discover Applied Sciences (Kerala, MCDA)

This is the paper closest to our study area, and its stated future work reads almost like a specification for Version 3.

| What the paper states | How Version 3 uses it |
| --- | --- |
| Future scope: “Future research should include temporal flooding dynamics.” | Direct hit. Our multi-epoch design and temporal hold-out are precisely this. |
| Future scope: “The model could be improved … [with] higher-resolution Digital Elevation Models.” | Direct hit. We replace SRTM with FABDEM v1.2, a bare-earth DEM in which buildings and forest canopy have been removed. |
| Future scope: “Further model performance improvement can be improved through cross-validation testing.” | Direct hit. We implement spatial block cross-validation by sub-watershed and temporal hold-out, and we report the naive random-split number alongside so the size of the inflation is visible. |
| Future scope: “exploring more advanced machine learning algorithms”. | We use a LightGBM + XGBoost ensemble under a positive-unlabeled framing. |
| Limitation: SRTM DEM has coverage gaps in mountainous terrain, and vegetation canopy interference causes vertical accuracy problems; further limits come from single-pass collection and fixed resolution. | Direct hit. FABDEM exists specifically to correct canopy and building bias in elevation data, so this named limitation is removed rather than inherited. |
| Reported result: AHP achieves AUC 0.763; LightGBM achieves 0.948, both under a conventional split. | We reproduce both as baselines and re-evaluate them under leak-free validation. This gives the panel a like-for-like comparison table (Section 22.4). |
| Future scope: coastal flooding mechanisms and climate-change scenarios. | Declared out of scope for one semester and listed as a future extension (Section 28). We prefer to say so than to promise it. |

# 16. Research Question (Version 3)

“Does the flood vulnerability of a plot of land change once it is built on, by how much, and can that change be predicted before construction happens — including the extra risk that new construction pushes onto the land downhill from it?”

Two sub-questions follow from it:

- RQ-a (spillover): does the amount of sealed surface draining into a cell predict flooding better than the amount of sealed surface on that cell, which is all the existing models use?

- RQ-b (validation honesty): how much of the reported accuracy of published Kerala flood susceptibility maps survives when the training and testing areas are kept geographically separate?

# 17. The Core Idea, in Plain Language

Every flood susceptibility map in our reference set is built the same way. Take the land as it looks today, take the places where flooding was seen in the past, and train a model to connect the two. This works well for the question “which existing neighbourhoods flooded?”

But our project promised a different feature: letting somebody check an empty plot before building on it. That is where the standard method quietly breaks, for two reasons.

Problem 1 — the plot changes when you build on it. The model reads the plot as it is now: bare soil or vegetation, water soaks in, low runoff, so it returns a reassuring answer. The moment a house, a compound wall and a paved yard exist, that same plot sheds water instead of absorbing it. The model was asked about the empty plot but the family will live on the built plot. The answer they were given does not describe the place they will actually be living in.

Problem 2 — your neighbour’s construction lands on you. Every one of these models treats each 10 m cell as if it were independent. Water does not work that way. If ten plots uphill of you are paved over, more water arrives at your gate, even though nothing changed on your own land. No model in our reference set can represent this at all, because none of them carries a feature describing what is upstream of a cell.

Version 3 fixes both. We compute, for every cell, how much sealed surface drains into it by following the actual downhill flow paths (we call this UICA). We then predict two numbers for every plot — one for the land as it is, one for the land as it would be once built on — and report the difference. Finally, we check this against reality using Kerala’s own recent history: plots that were empty before the 2018 flood and have been built on since.

# 18. Novelty of the Proposed Work

Each point below names what the existing paper does not do, what we implement instead, and who benefits.

## 18.1 Against the base paper — Mohamadiazar et al. (2024)

- They do not do this: their model needs live rainfall as an input, so it can only answer during or just after a storm. We are implementing: a standing, storm-independent vulnerability map that can be queried on any ordinary day. Why it helps ordinary people: a family deciding on a plot in February cannot wait for the next flood to find out whether the land is safe.

- They do not do this: their output is a flooded / not-flooded pixel mask with no reasoning attached. We are implementing: a graded score with a per-factor breakdown and a plain-language counterfactual — “this would drop from High to Moderate if paved cover were kept under about 42 %”. Why it helps ordinary people: a warning tells you to be afraid; a counterfactual tells you what to do.

- They do not do this: they predict only for land as it currently stands. We are implementing: a second prediction for the same cell as if it were built on. Why it helps ordinary people: it is the only version of the number that is actually relevant to somebody who is about to build.

## 18.2 Against the explainable-ML paper — Antzoulatos et al. (2022)

- They do not do this: their labels come from a calibrated 2D hydraulic model that supplies water depth and velocity — infrastructure that does not exist for the Periyar basin, and which they themselves flag as the field’s crucial bottleneck. We are implementing: positive-unlabeled learning, which needs only confirmed flooded locations and never has to pretend it knows which places are safe. Why it helps ordinary people: it means the method can be deployed in ordinary Indian districts that will never have a calibrated hydraulic model.

- They do not do this: their dynamic tool responds to a live flood event. We are implementing: a tool that responds to land-use change — the slow process that is actually raising Kochi’s flood risk year on year. Why it helps ordinary people: most of the risk increase happens between floods, not during them, and nobody is currently watching that.

## 18.3 Against the Pakistan framework — Waleed & Sajjad (2025)

- They explicitly list this as future work they did not do: integrating land-use change such as urban sprawl into the susceptibility framework. We are implementing: exactly that, through ΔUICA and the development-aware prediction stage. Why it helps ordinary people: it turns a stated research gap into a usable warning about where growth is making things worse.

- They do not do this: their model is per-pixel, so it cannot express that one person’s construction raises another person’s risk. We are implementing: flow-routed upslope features, so the model can attribute risk to upstream development. Why it helps ordinary people: for the first time a resident can see that their flooding is being caused by construction uphill — which is information a ward council can actually act on.

- They do not do this: their susceptibility values come without any statement of confidence, and the map covers the whole country uniformly. We are implementing: conformal prediction intervals plus an Area-of-Applicability mask that greys out places where the model is extrapolating. Why it helps ordinary people: an honest “we do not know here” is far safer than a confident wrong colour on a map somebody uses to buy land.

## 18.4 Against the Kerala MCDA paper — Kendagannaswamy et al. (2025)

- They do not do this: they validate with a conventional split, which lets neighbouring pixels appear in both training and testing and inflates the score. Their own conclusion asks for cross-validation testing as future work. We are implementing: spatial block cross-validation by sub-watershed plus a temporal hold-out, reporting the naive number alongside so the gap is visible. Why it helps ordinary people: it tells the public how much to trust a published flood map before anyone makes a decision on it.

- They do not do this: their scoring uses expert-assigned AHP weights, which they themselves show performs worse (AUC 0.763) than a learned model (0.948). We are implementing: learned weights under a PU framing, with AHP kept only as a reproduced baseline. Why it helps ordinary people: the number they are shown comes from evidence rather than from opinion.

- They do not do this: they use SRTM elevation and note that tree canopy and buildings corrupt its vertical accuracy. We are implementing: FABDEM, a bare-earth DEM with canopy and buildings removed. Why it helps ordinary people: in a green, densely built district like Kochi, a DEM that mistakes a coconut canopy for ground level puts the low-lying plots in the wrong class.

- They do not do this: their map is a single static snapshot of Kerala. We are implementing: a multi-epoch design across the 2018, 2019 and 2021 events that measures whether a model still works three years after it was fitted. Why it helps ordinary people: it establishes how often such a map must be refreshed before it starts misleading people.

## 18.5 Summary of what is genuinely new

| # | Novel element | Nobody in the four papers does this |
| --- | --- | --- |
| N1 | Prospective (development-aware) prediction — two susceptibility values per cell, current and post-development, and the uplift Δ between them | All four predict only for land as it currently stands |
| N2 | UICA — Upslope Impervious Contributing Area, flow-routed sealed surface draining into a cell, plus its change ΔUICA over time | All four use only local imperviousness or NDVI; none routes it downhill |
| N3 | Natural-experiment validation using plots that were empty before 2018 and built on by 2021, as treatment against matched still-empty controls | None validates a susceptibility model against observed development |
| N4 | Leak-free evaluation protocol — spatial block CV plus temporal hold-out, with the inflated random-split number reported side by side | All four use a conventional split; the Kerala paper lists this as future work |
| N5 | Constrained counterfactual recourse — only buildable levers may move, elevation and slope are fixed | Explanation stops at SHAP attribution; no actionable, constraint-respecting advice |
| N6 | Positive-unlabeled label model — never assumes an unflooded cell is safe | All four treat sampled non-flood points as true negatives |

# 19. System Architecture (Version 3)

The pipeline below replaces the Version 2 block diagram in Section 5. Blue blocks are standard practice carried over from the reference papers; amber blocks marked with a star are new in Version 3.

Figure V3-1 — End-to-end architecture of the development-aware, flow-coupled prospective flood vulnerability framework.

How to read the diagram. Stage 1 gathers and aligns free satellite and map data. Stage 2 splits into two tracks: one builds the record of where flooding actually happened, the other builds the record of where construction actually happened. Stage 3 converts terrain into hydrology-aware features, including our new upslope features. Stage 4 trains the model and — critically — tests it honestly. Stage 5 is the research core: predicting what a plot becomes once it is built on. Stage 6 turns the number into advice for a planner and for a citizen.

# 20. Stage-by-Stage Implementation Plan

## 20.1 Stage 1 — Data acquisition and harmonisation

What we do. Pull every layer for the Kochi / Periyar basin study area, reproject everything to UTM Zone 43N, and resample onto one shared 10 m grid so that every layer lines up cell for cell. Coarser layers (30 m DEM, 5 km rainfall) are resampled up and are honestly described as context around a cell, not as 10 m measurements.

Epochs. We build the full stack for five dates: 2017 (pre-flood baseline), 2018, 2019, 2021 (three flood events) and 2024 (current state).

Key change from Version 2. The DEM is FABDEM v1.2 rather than SRTM or plain Copernicus GLO-30. FABDEM is the same Copernicus DEM with forest canopy and buildings machine-learned out of it. This matters in Kochi, where dense coconut canopy sits over exactly the low-lying land we care about, and it directly answers the canopy-interference limitation the Kerala paper reports.

## 20.2 Stage 2A — Multi-epoch flood inventory

What we do. For each flood event, take Sentinel-1 SAR scenes from the flood window, apply the standard chain (radiometric calibration → speckle filtering → terrain correction), convert backscatter to decibels, and threshold with Edge-Otsu to obtain a water mask. This is the same procedure as the base paper, and we adopt it deliberately so our inventory is comparable.

What we add. We subtract the JRC Global Surface Water permanent-water layer so that rivers and backwaters are not counted as flooding, and we build three independent inventories (2018, 2019, 2021) instead of one. DFO polygons, Copernicus EMS / UNOSAT rapid-mapping products for the 2018 activation, and local records serve as corroboration, exactly as set out in Section 6.

Why three events matter. One event gives one map. Three events let us ask whether a model fitted on one still works on the next — which is the temporal test the Kerala paper asked for.

## 20.3 Stage 2B — Built-up change detection

What we do. Segment land cover for each epoch into five classes (building, vegetation, water, impervious surface / road, bare land) using DeepLabV3+ or U-Net with an ImageNet-pretrained encoder, fine-tuned on Kochi tiles. Google Dynamic World and ESA WorldCover provide weak labels to bootstrap training, and OSM / Microsoft / Google Open Buildings footprints provide building labels.

What it is for. Differencing the epochs gives every cell a build year — the epoch at which it changed from unbuilt to built. This is not a decorative overlay. It produces the treatment group (cells empty before 2018, built by 2021) and the control group (cells that stayed empty) that Stage 5 depends on, and it drives the ΔUICA feature in Stage 3.

Honesty note carried over from Version 2. We work at cluster / grid-cell level, not at individual-building level, because a single house is often smaller than one Sentinel-2 pixel.

## 20.4 Stage 3 — Flow-coupled feature engineering

Standard factors. From FABDEM: fill sinks, compute D8 (or D-infinity) flow direction and flow accumulation, then derive elevation, slope, aspect, curvature, Topographic Wetness Index (TWI) and HAND (Height Above Nearest Drainage). From vector data: Euclidean distance to river, to drainage line and to road. From CHIRPS: rainfall frequency, counted as the number of days above 10 mm across 2010–2024. From Stage 2B: local built-up fraction and vegetation fraction.

HAND deserves a note — it measures how high a cell sits above the nearest stream it drains into, and it is a strong flood predictor that none of our four reference papers uses.

The new feature: UICA (Upslope Impervious Contributing Area). Ordinary flow accumulation counts how many cells drain into a cell. UICA counts the same thing but weights each upslope cell by how sealed it is:

UICA(c)  =  Σ over all cells u that drain into c of  [ impervious_fraction(u) × cell_area ]

In words: “how much paved, roofed or otherwise sealed surface sends its water down to this cell.” It is computed with a weighted flow-accumulation routine (WhiteboxTools or pysheds both support a weights raster), so it costs one extra raster operation and no extra data.

The second new feature: ΔUICA. The change in UICA between two epochs. This is the additional runoff loaded onto a cell purely because new construction appeared uphill of it. It is the quantity that connects Stage 2B to the model as a genuine causal input.

## 20.5 Stage 4 — Positive-unlabeled learning with leak-free validation

The label problem. Every reference paper samples “flood” points inside the observed flood extent and “non-flood” points outside it, then trains a normal classifier. But a cell outside the observed extent is not proven safe. SAR misses flooding under tree canopy and in urban radar shadow, and satellites only pass every few days, so a place may simply never have been imaged while it was under water. Treating those cells as confirmed negatives teaches the model something false.

What we do instead. Flooded cells are positive; everything else is unlabeled. We use PU-bagging with an Elkan–Noto style class-prior correction to recover calibrated probabilities, then train a LightGBM + XGBoost ensemble.

Leak-free validation. Neighbouring raster cells are nearly identical, so a random train/test split effectively puts the test data into the training set and inflates the score. We therefore evaluate three ways and report all three:

- Random split — the conventional method, reported only so the inflation is visible.

- Spatial block cross-validation — folds are whole HydroBASINS level-12 sub-watersheds, so training and testing areas never touch.

- Temporal hold-out — fit on the 2018 event, test on 2019 and on 2021.

Trust layer. Every predicted cell carries a SHAP factor attribution, a conformal prediction interval, and an Area of Applicability flag that greys out cells whose feature values fall outside the range the model actually learned from.

## 20.6 Stage 5 — Prospective, development-aware prediction (the research core)

Dual prediction. For every cell we produce two values:

- S_current — susceptibility given the land cover as it stands today. This is what all four reference papers produce.

- S_developed — susceptibility for the same cell with its land-cover features set to a built state: impervious fraction raised to the observed Kochi residential norm, vegetation fraction reduced accordingly, and the UICA of every downstream cell updated to account for the new sealed surface.

The uplift.  Δ = S_developed − S_current. This is precisely the amount that a naive “check before you build” lookup silently omits. We report Δ per parcel, aggregated per ward, and as a cumulative build-out scenario in which every currently-empty buildable cell in a sub-basin is developed at once.

Validation by natural experiment. This is the experiment that makes the claim testable rather than merely plausible:

- Treatment group: cells that were unbuilt before the 2018 event and had been built on by 2021, identified by Stage 2B.

- Control group: cells that remained unbuilt, matched to the treatment cells on elevation, slope, HAND and distance to road by propensity matching.

- The test: fit the model on pre-2018 data only. For the treatment cells, does S_developed predict their post-construction flood outcome in 2021 better than S_current does? We report the difference in ROC-AUC and PR-AUC, and the percentage of cells that shift vulnerability class.

Stated honestly: this is a quasi-experimental design, not a randomised one. People choose to build on flatter, better-connected land, so treatment and control cells differ systematically. Propensity matching reduces this but does not remove it, and we will describe results as association under matching, never as a proven causal effect. We expect this to be the panel’s sharpest question and we intend to have the answer ready.

## 20.7 Stage 6 — Counterfactual recourse and dual dashboard

Constrained counterfactuals. A SHAP value says “elevation pushed this score up”, which a homeowner cannot act on. A counterfactual says what would have to change. We therefore split the features:

- Mutable (a builder can change these): impervious fraction, vegetation fraction, setback from the drainage line, on-plot retention storage.

- Immutable (nobody can change these): elevation, slope, HAND, distance to the river.

We then search for the smallest change in the mutable features that moves the cell into a lower vulnerability class, and express it in plain language: “High → Moderate if paved cover is kept under about 42 % and a 3 m drainage setback is maintained.” Because immutable features are frozen, the advice is always physically achievable — which generic counterfactual methods do not guarantee.

Planner view. Ward-level vulnerability with the Area-of-Applicability mask, a ΔUICA hotspot map showing where upstream growth is loading risk downstream, the build-out scenario simulator, and a ranked list of the parcels where new construction would harm neighbours most.

Citizen view. Search any location, including empty land. The answer gives the current class, the uplift Δ that would apply if the plot were built on, the main contributing factors in plain words, the confidence interval, and the top three actions that would lower the class.

# 21. Dataset Availability and Access Check

Every layer below is free and publicly accessible, and the great majority are available directly inside Google Earth Engine, which removes almost all manual downloading. Nothing in this plan depends on paid data, on a commercial licence, or on a dataset that must be requested from an agency.

| Layer | Dataset | Resolution | How we get it | Status |
| --- | --- | --- | --- | --- |
| Optical imagery | Sentinel-2 Level-2A | 10 m | GEE: COPERNICUS/S2_SR_HARMONIZED | Free, confirmed |
| Radar imagery | Sentinel-1 GRD (VV + VH) | 10 m | GEE: COPERNICUS/S1_GRD; Copernicus Data Space | Free, confirmed |
| Elevation (primary) | FABDEM v1.2 (bare-earth) | 30 m | University of Bristol data repository (data.bris); free for academic use | Free, academic licence |
| Elevation (fallback) | Copernicus GLO-30 | 30 m | GEE / OpenTopography | Free, confirmed |
| Rainfall | CHIRPS Daily | ~5 km | GEE: UCSB-CHG/CHIRPS/DAILY | Free, confirmed |
| Land cover (weak labels) | Google Dynamic World | 10 m | GEE: GOOGLE/DYNAMICWORLD/V1 | Free, confirmed |
| Land cover (alternative) | ESA WorldCover v200 | 10 m | GEE: ESA/WorldCover/v200 | Free, confirmed |
| Permanent water mask | JRC Global Surface Water | 30 m | GEE: JRC/GSW1_4/GlobalSurfaceWater | Free, confirmed |
| Sub-watershed blocks | HydroBASINS level 12 | vector | hydrosheds.org | Free, confirmed |
| Rivers and drainage | HydroRIVERS + OpenStreetMap | vector | hydrosheds.org / Geofabrik India extract | Free, confirmed |
| Roads | OpenStreetMap (or GRIP) | vector | Geofabrik India extract | Free, confirmed |
| Building footprints | Microsoft Global ML Building Footprints; Google Open Buildings v3 | vector | GitHub release / GEE | Free, confirmed |
| Population | WorldPop constrained | 100 m | worldpop.org / GEE | Free, confirmed |
| Flood evidence 1 | Dartmouth Flood Observatory, 2018 Kerala | ~250 m | DFO archive | Free, confirmed; coarse (see §6.1) |
| Flood evidence 2 | Copernicus EMS / UNOSAT rapid mapping, Aug 2018 Kerala | vector | emergency.copernicus.eu / unosat.org | Free — activation ID to be confirmed in Week 1 |
| Administrative units | Kerala LSGD ward / panchayat boundaries | vector | Kerala State Spatial Data Infrastructure / OSM | Free — coverage to be confirmed in Week 1 |

Two items are marked for Week-1 confirmation rather than assumed. We would rather flag them now than discover a gap in Week 8. Neither is on the critical path: the Copernicus EMS layer is a third corroboration source and the analysis runs without it, and ward boundaries affect only how results are aggregated for the planner view.

## 21.1 The one genuine go/no-go check

Sentinel-1 scene availability during the August 2018 flood peak. Our temporal design assumes usable SAR acquisitions over the Periyar basin while flooding was actually present. Sentinel-1 revisits every 6 days at that latitude and the 2018 Kerala flooding persisted for roughly two weeks, so at least two passes are expected — but this must be verified before anything else is built. If the scenes are not usable, we fall back to the DFO and Copernicus EMS polygons as the 2018 inventory and lean more heavily on the 2019 and 2021 events. This check is Week 1, Task 1.

# 22. Evaluation Plan (Version 3)

## 22.1 Segmentation and change detection

Unchanged from Version 2 Section 7.1: Pixel Accuracy, Precision, Recall, F1, IoU and Dice, per class. Change detection is verified by manual sampling against Google Earth historical imagery, reporting Precision / Recall / F1 / IoU on the verified sample, with the absence of a formal benchmark disclosed openly.

## 22.2 Susceptibility model

Primary metrics: PR-AUC (flooded cells are a minority class, so this is more informative than ROC-AUC alone), ROC-AUC, F1 at the operating threshold, Brier score for calibration, and conformal interval coverage. Every metric is reported under all three validation schemes so the effect of the protocol is visible.

## 22.3 The four novelty experiments

| ID | Experiment | What it tests | The headline number we report |
| --- | --- | --- | --- |
| E1 | Validation inflation. One model, three validation schemes: random split, spatial block CV, temporal hold-out. | RQ-b — how much published accuracy survives honest evaluation. | The drop in ROC-AUC from random split to spatial block CV, in AUC points. |
| E2 | Spillover. Model with local imperviousness only, versus with UICA, versus with UICA + ΔUICA, all under spatial block CV. | RQ-a — whether upslope sealed surface carries signal beyond local sealed surface. | The gain in PR-AUC from adding UICA, with a permutation test for significance. |
| E3 | Prospective prediction (natural experiment). Fit on pre-2018 data; compare S_current against S_developed at predicting the 2021 outcome of cells built between 2018 and 2021, against matched still-empty controls. | The central research question — is a pre-construction lookup systematically wrong? | The under-estimation, in susceptibility points and in the percentage of cells that shift vulnerability class. |
| E4 | Recourse validity. For every High and Very-High parcel, search for a counterfactual within the buildable constraints. | Whether the advice the dashboard gives is actually achievable. | The percentage of high-risk parcels for which a feasible counterfactual exists. |

## 22.4 Baseline comparison table (to be filled with our results)

This is the single most important table in the final report. It places our method beside the published Kerala result under identical, leak-free evaluation. We reproduce the baselines ourselves rather than quoting them, so the comparison is like for like.

| Model | Validation | ROC-AUC | PR-AUC | Note |
| --- | --- | --- | --- | --- |
| AHP weighted sum (reproduces Kendagannaswamy et al.) | Random split | — | — | Published value ≈ 0.763 |
| LightGBM, standard factors (reproduces Kendagannaswamy et al.) | Random split | — | — | Published value ≈ 0.948 |
| LightGBM, standard factors | Spatial block CV | — | — | Expected to drop — this is finding E1 |
| LightGBM + HAND + UICA | Spatial block CV | — | — | Finding E2 |
| LightGBM + HAND + UICA + ΔUICA | Spatial block CV | — | — | Finding E2 |
| PU-LightGBM/XGBoost + UICA + ΔUICA | Spatial block CV | — | — | Proposed model |
| PU-LightGBM/XGBoost + UICA + ΔUICA | Temporal hold-out 2018 → 2021 | — | — | Hardest test; expected lowest and most honest number |

A note on what a good result looks like. We are not aiming to beat 0.948. Under leak-free validation we expect lower numbers than the published ones, and that is the finding, not a failure. A defensible 0.85 under spatial block CV is worth more than an inflated 0.95 under a leaky split, and saying so clearly is part of the contribution.

# 23. Ablation Study (Version 3)

This replaces the Version 2 ablation in Section 8. Every case is evaluated under spatial block cross-validation, so each row isolates one design decision. Cases A5 to A7 correspond one-to-one with our novelty claims.

| Case | Features included | What it isolates |
| --- | --- | --- |
| A0 | Elevation only | Absolute floor; the simplest possible model |
| A1 | + slope, aspect, curvature, TWI, HAND | Terrain-only baseline |
| A2 | + distance to river, drainage line, road | Proximity factors |
| A3 | + rainfall frequency (CHIRPS) | Climate forcing |
| A4 | + local imperviousness and vegetation fraction | This row matches what the four reference papers use — our comparison point |
| A5 | + UICA (flow-routed upslope sealed surface) | Novelty N2, part 1 — does spillover carry signal beyond local land cover? |
| A6 | + ΔUICA (change in upslope sealed surface over time) | Novelty N2, part 2 — does the growth signal add anything further? |
| A7 | A6 retrained under positive-unlabeled learning | Novelty N6 — does correcting the label assumption change the answer? |

Ablation results are also used as a cross-check on SHAP. Waleed & Sajjad warn that explainable-AI feature rankings carry context-dependent biases, so we treat a factor as genuinely important only when SHAP and the ablation agree.

# 24. Feasibility, Tools and Compute

| Component | Tools | Compute need |
| --- | --- | --- |
| Data preparation, SAR processing, compositing | Google Earth Engine (Python API) | Free tier; runs in minutes |
| Flow routing, UICA, HAND, TWI | WhiteboxTools or pysheds (both support a weights raster for flow accumulation, which UICA requires); rasterio / GDAL | CPU only; minutes on a basin-sized raster |
| Land-cover segmentation and change detection | PyTorch + segmentation_models_pytorch (DeepLabV3+ / U-Net) | One GPU; Google Colab or Kaggle free tier is sufficient |
| Susceptibility model | LightGBM, XGBoost, scikit-learn | CPU only |
| PU learning | PU-bagging implemented over scikit-learn; Elkan–Noto correction | CPU only |
| Spatial CV and Area of Applicability | geopandas + custom folds over HydroBASINS | CPU only |
| Uncertainty | MAPIE (conformal prediction) | CPU only |
| Explanation | SHAP; DiCE for counterfactuals, with immutable features frozen | CPU only |
| Dashboard | Streamlit or Flask + Leaflet / Folium | Laptop |

Why this is achievable in one semester. The study area is a single basin, not a state or a country. The heavy deep-learning component is confined to Stage 2B and can fall back to Dynamic World labels if training time runs short. Everything downstream of Stage 3 is gradient boosting on a tabular dataset, which trains in seconds. The genuinely new work — UICA, the dual prediction and the natural experiment — is raster arithmetic and careful experimental design, not expensive computation.

# 25. Work Plan

| Weeks | Phase | Deliverable | Gate |
| --- | --- | --- | --- |
| 1 | Go / no-go checks | Sentinel-1 scene availability for Aug 2018 verified; Copernicus EMS activation and ward boundaries located; study-area extent frozen | If SAR scenes are unusable, switch to the polygon-inventory fallback now |
| 2–3 | Data pipeline | All layers pulled and aligned on the 10 m grid for all five epochs | Stack renders |
| 4–5 | Flood inventory (Stage 2A) | Three event inventories, permanent water removed, cross-checked against DFO | Inventory agrees with DFO extent within a stated tolerance |
| 6–8 | Segmentation and change detection (Stage 2B) | Five-class maps per epoch; build-year raster; treatment and control groups defined | IoU acceptable on held-out tiles; treatment group large enough for E3 |
| 8–9 | Flow-coupled features (Stage 3) | HAND, TWI, UICA and ΔUICA rasters | UICA visually tracks known drainage lines |
| 10–11 | Model and honest validation (Stage 4) | PU ensemble trained; all three validation schemes run; experiment E1 complete | E1 result obtained — first novel finding banked |
| 11–12 | Ablation and E2 | Cases A0–A7 complete; UICA contribution quantified | E2 result obtained |
| 13–14 | Prospective prediction (Stage 5) | S_current and S_developed maps; uplift Δ; experiment E3 complete | Core research claim tested |
| 15 | Recourse and dashboard (Stage 6) | Counterfactual engine; planner and citizen views; E4 complete | Demo runs |
| 16 | Writing | Final report, baseline comparison table, presentation | Submission |

Deliberate sequencing note. Experiment E1 lands in Week 11, well before the end. If the more ambitious Stage 5 runs into trouble, we still hold a complete, defensible novel result about validation honesty in the Kerala context. The plan is designed so that we cannot finish the semester empty-handed.

# 26. Risks and Mitigation

| Risk | Impact | Mitigation |
| --- | --- | --- |
| Sentinel-1 scenes unusable at the 2018 flood peak | Breaks the multi-epoch design | Week-1 go/no-go check; fall back to DFO and Copernicus EMS polygons as the 2018 inventory and rely more on 2019 and 2021 |
| Too few cells changed from empty to built between 2018 and 2021 | Experiment E3 is underpowered | Widen the window to 2016–2024; widen the study area to the full Periyar basin; report a power analysis rather than a weak result |
| Confounding in the natural experiment — people build on better land | Undermines any causal reading of E3 | Propensity matching on elevation, slope, HAND and road distance; report as association under matching, never as a causal effect; state the limitation in the report and the viva |
| Persistent monsoon cloud in Sentinel-2 | Gaps in land-cover segmentation | Dry-season (December–February) median compositing; Sentinel-1 fusion; Dynamic World as fallback labels |
| FABDEM licence or access problem | Loss of the bare-earth advantage | Copernicus GLO-30 fallback, with the canopy-bias limitation then stated openly |
| Deep-learning segmentation takes longer than budgeted | Stage 2B slips and delays everything downstream | Dynamic World and WorldCover can supply land cover directly; the segmentation model is an accuracy improvement, not a dependency |
| Severe class imbalance (few flooded cells) | Misleading accuracy figures | PR-AUC as the primary metric; PU framing; stratified spatial folds |
| Google Earth Engine quota limits on ML | Cannot train inside GEE | Prepare data in GEE, export, train locally — the same split Waleed & Sajjad adopted |

# 27. Limitations and Ethics We State Openly

Version 2’s limitations in Section 9 all still stand. Version 3 adds the following, and we intend to state them in the report and the viva rather than wait to be asked.

- The natural experiment is quasi-experimental. People choose where to build, so treatment and control cells are not exchangeable. Matching narrows the gap but does not close it. We report association under matching, never a proven causal effect.

- Counterfactuals are what-if statements under our model, not engineering guarantees. “High → Moderate if paved cover stays under 42 %” describes how the model responds, not a certified hydraulic outcome. It is guidance, not a building permit.

- S_developed uses a typical built density, not a specific building plan. We set the impervious fraction to an observed Kochi residential norm. A particular plot may be built more or less densely than that.

- UICA is a runoff proxy, not a drainage model. It follows terrain flow paths and ignores stormwater pipes, culverts and pumping. In a serviced urban area the real routing may differ substantially.

- Rainfall is treated as a long-run frequency, not an event. We deliberately avoid the base paper’s rainfall-timing problem, but the cost is that our map cannot say anything about any specific storm.

- Flood risk remains multi-causal. River discharge, tides, soil permeability and drainage maintenance all matter and are outside our data.

- Ethics. Following the concern Waleed & Sajjad raise, a public vulnerability map can be misused — to depress land prices unfairly, to justify unequal service provision, or to discriminate against particular localities. Our mitigations are to publish the Area-of-Applicability mask so uncertain areas are visibly marked, to report confidence intervals rather than bare classes, to use relative vulnerability language rather than implied probabilities of loss, and to state that the output is advisory and not a substitute for a statutory survey.

# 28. Scope — Core Deliverable and Extensions

Core deliverable (must be finished): multi-epoch flood inventory for the Periyar basin; built-up change detection with build years; flow-coupled features including UICA; a PU-trained gradient-boosting susceptibility model evaluated under spatial block CV and temporal hold-out; experiments E1 and E2; the baseline comparison table; and an interactive map.

- Extension 1: the full prospective dual prediction and experiment E3 — the highest-value extension, targeted for Weeks 13–14.

- Extension 2: constrained counterfactual recourse and experiment E4.

- Extension 3: conformal prediction intervals and the Area-of-Applicability mask.

- Extension 4: the citizen-facing dashboard with plain-language explanation.

- Extension 5 (explicitly out of scope): coastal and tidal flooding mechanisms and climate-change scenarios, both named as future work by the Kerala paper. We state plainly that one semester does not accommodate them.

# 29. One-Paragraph Summary (Version 3)

“We build a flood vulnerability mapping framework for the Kochi / Periyar basin that answers a question existing flood susceptibility maps cannot: what happens to a plot of land once it is built on. Existing models — including the published Kerala susceptibility map — read land cover as it stands today, treat every pixel as independent, assume that anywhere flooding was not observed is safe, and validate on randomly split data that lets neighbouring pixels leak between training and testing. We change all four. We route sealed surface downhill to compute how much runoff drains into each cell (UICA) and how that has grown with new construction (ΔUICA); we train under a positive-unlabeled framing that never assumes an unflooded cell is safe; we evaluate with spatial block cross-validation and a temporal hold-out across the 2018, 2019 and 2021 events, reporting the inflated conventional number alongside so the difference is visible; and we predict two susceptibility values per cell — as it is, and as it would be once developed — validating the difference against plots in Kochi that were empty before 2018 and have been built on since. The result is delivered as constrained, achievable advice for a planner and for a citizen checking a plot before building. All data is free and public, and three of our four reference papers name this direction as their own stated future work.”

# 30. References

Primary reference papers (supplied)

- Mohamadiazar, N., Ebrahimian, A. & Hosseiny, H. (2024). Integrating deep learning, satellite image processing, and spatial-temporal analysis for urban flood prediction. *Journal of Hydrology*, 639, 131508.

- Antzoulatos, G. et al. (2022). Flood Hazard and Risk Mapping by Applying an Explainable Machine Learning Framework Using Satellite Imagery and GIS Data. *Sustainability*, 14(6), 3251.

- Waleed, M. & Sajjad, M. (2025). High-resolution flood susceptibility mapping and exposure assessment in Pakistan: An integrated artificial intelligence, machine learning and geospatial framework. *International Journal of Disaster Risk Reduction*, 121, 105442.

- Kendagannaswamy, M. S., Roopa, C. K., Harish, B. S. & Mukesh, M. S. (2025). Multi-criteria decision analysis for regional-scale flood susceptibility mapping in Kerala state, India. *Discover Applied Sciences*, 7, 598.

Methodological references supporting the Version 3 design

- Roberts, D. R. et al. (2017). Cross-validation strategies for data with temporal, spatial, hierarchical, or phylogenetic structure. *Ecography*, 40(8), 913–929. — the basis for spatial block cross-validation.

- Ploton, P. et al. (2020). Spatial validation reveals poor predictive performance of large-scale ecological mapping models. *Nature Communications*, 11, 4540. — evidence that random splits inflate accuracy on spatial data.

- Meyer, H. & Pebesma, E. (2021). Predicting into unknown space? Estimating the area of applicability of spatial prediction models. *Methods in Ecology and Evolution*, 12(9), 1620–1633. — the Area-of-Applicability mask.

- Wadoux, A. M. J.-C., Heuvelink, G. B. M., de Bruin, S. & Brus, D. J. (2021). Spatial cross-validation is not the right way to evaluate map accuracy. *Ecological Modelling*, 457, 109692. — the counter-argument, which we cite deliberately: there is a live methodological debate here, and engaging with both sides is part of the contribution.

- Elkan, C. & Noto, K. (2008). Learning classifiers from only positive and unlabeled data. *KDD ’08*. — the class-prior correction used in Stage 4.

- Bekker, J. & Davis, J. (2020). Learning from positive and unlabeled data: a survey. *Machine Learning*, 109, 719–760.

- Mothilal, R. K., Sharma, A. & Tan, C. (2020). Explaining machine learning classifiers through diverse counterfactual explanations. *FAT* ’20*. — the DiCE method behind Stage 6.

- Lundberg, S. M. & Lee, S.-I. (2017). A unified approach to interpreting model predictions. *NeurIPS*. — SHAP.

- Angelopoulos, A. N. & Bates, S. (2021). A gentle introduction to conformal prediction and distribution-free uncertainty quantification. — the conformal intervals in Stage 4.

- Hawker, L. et al. (2022). A 30 m global map of elevation with forests and buildings removed. *Environmental Research Letters*, 17, 024016. — FABDEM.

- Nobre, A. D. et al. (2011). Height Above the Nearest Drainage — a hydrologically relevant new terrain model. *Journal of Hydrology*, 404(1–2), 13–29. — HAND.

- Adger, W. N. (2006). Vulnerability. *Global Environmental Change*, 16(3), 268–281. — the exposure / sensitivity / adaptive-capacity framing cited by Waleed & Sajjad and used in our honesty statement.

— End of Version 3 —